# Workshop Setup: Configuring the Raspberry Pi 5

This guide provides a detailed, step-by-step procedure for setting up a Raspberry Pi 5 running Raspberry Pi OS (Bookworm, 64-bit) for use with the Seeed Studio Grove Base Hat, focusing on preventing known compatibility issues.Hardware and Software Prerequisites

- Hardware: Raspberry Pi 5, Seeed Studio Grove Base Hat

- Operating System (OS): Raspberry Pi OS (Bookworm, 64-bit)

-----Part 1: System Preparation and Interface Enabling

This section involves a one-time configuration of the system to enable the necessary hardware interfaces. This is critical for peripherals, such as an LCD, to function correctly.

- Open the Terminal:

  - Use the keyboard shortcut: Ctrl+Alt+T

- Enable Hardware Interfaces via raspi-config:

  - Run the configuration utility:
sudo raspi-config

  - Navigate through the menu options:

    - Select 3 Interface Options.

    - Select I5 I2C (Inter-Integrated Circuit) and choose Yes to enable it. (Required for many Grove modules, including the LCD).

    - (Optional but Recommended) Select I4 SPI (Serial Peripheral Interface) and choose Yes to enable it.

  - Select Finish.

  - When prompted, select Reboot to apply the changes.

-----Part 2: Creating a Python Virtual Environment

The Raspberry Pi 5 platform requires the use of a Python Virtual Environment (venv) to isolate project dependencies and prevent potential system crashes caused by conflicting package versions. All subsequent work will be done within this environment.

- Create the Project Folder:

Create a directory for the workshop files and navigate into it:
mkdir workshop

  - cd workshop

- Create the Virtual Environment:

  - Create the venv named env, ensuring it can access existing system-wide packages:
python3 -m venv env --system-site-packages

- Activate the Environment:

  - Crucial: This command must be run every time you open a new terminal window to work on the project.
source env/bin/activate

  - Verification: Your terminal prompt should now start with (env) indicating that the virtual environment is active.

-----Part 3: Installing Drivers (The "Golden Sequence")

This specific installation order is necessary to successfully install the Grove Python library while bypassing a known issue with the Pi 5 ("SOC peripheral" crash) by swapping out a broken dependency.Step A: Install the Grove Library from Source

This step downloads the full Grove Python library, which includes necessary drivers (like those for the LCD) that are often missing from the standard PyPI release.

- Install Git (if missing):

  - Ensure the git package is installed to clone from the repository:
sudo apt install git -y

- Download and Install the Full Library:

  - Install the library directly from the Seeed-Studio GitHub repository:
pip install git+https://github.com/Seeed-Studio/grove.py

Step B: The "Pi 5 Fix" (Compatibility Patch)

The previous installation step often introduces an old, incompatible version of the RPi.GPIO driver that causes crashes on the Raspberry Pi 5. This step removes the broken driver and installs a stable compatibility layer.

- Remove the Broken Driver:

  - Uninstall the conflicting RPi.GPIO package:
pip uninstall RPi.GPIO -y

- Install the Compatibility Driver:

  - Install the rpi-lgpio package, which provides stable GPIO access for the Pi 5:
pip install rpi-lgpio

Your Raspberry Pi 5 setup is now complete and configured to run Grove Hat projects within the virtual environment.